VAG-COM, Meetwaardeblokken Module 01 (Engine)        Erik-Jan Huijbrechts

Group 000, General Engine Data, (no normalization)

| 000 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 1 Bank-Systems |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Lambda control learning value partial load |  |
|  |  |  |  |  |  |  |  |  | Lambda control learning value idle |  |  |
|  |  |  |  |  |  |  |  | Lambda control |  |  |  |
|  |  |  |  |  |  |  | Idle Air Control Valve learning value |  |  |  |  |
|  |  |  |  |  |  | Idle Air Control Valve |  |  |  |  |  |
|  |  |  |  |  | Throttle valve potentiometer |  |  |  |  |  |  |
|  |  |  |  | Voltage |  |  |  |  |  |  |  |
|  |  |  | RPM |  |  |  |  |  |  |  |  |
|  |  | Load |  |  |  |  |  |  |  |  |  |
|  | Engine coolant temperature |  |  |  |  |  |  |  |  |  |  |
| 000 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 2 Bank-Systems |
|  |  |  |  |  |  |  |  |  |  | Lambda adaptation (add) Bank 2 |  |
|  |  |  |  |  |  |  |  |  | Lambda adaptation (add) Bank 1 |  |  |
|  |  |  |  |  |  |  |  | Lambda control Bank 2 |  |  |  |
|  |  |  |  |  |  |  | Lambda control Bank 1 |  |  |  |  |
|  |  |  |  |  |  | Idle Air Control Valve learning value |  |  |  |  |  |
|  |  |  |  |  | Idle Air Control |  |  |  |  |  |  |
|  |  |  |  | Throttle valve angle |  |  |  |  |  |  |  |
|  |  |  | RPM |  |  |  |  |  |  |  |  |
|  |  | Load |  |  |  |  |  |  |  |  |  |
|  | Engine coolant temperature |  |  |  |  |  |  |  |  |  |  |

Groups 001 - 009 General Engine Data

| 001 | General |  | 2 Bank systems |  |
| --- | --- | --- | --- | --- |
|  | RPM | Engine coolant temperature | Lambda control value Bank 1 | Lambda control value Bank 2 |
|  | [1/min] | [°C] | [%] | [%] |
| 001 | General |  | 1-Bank-Systems |  |
|  | RPM | Engine coolant temperature | Lambda control value (Injection adjustment) | Adjustment requirements for basic setting |
|  | [1/min] | [°C] | [%] | xxxxxxxx │││││││└Coolant │││││││ temp. > 80°C ││││││└─RPM < ││││││ 2000 1/min │││││└─ Throttle │││││ closed ││││└── Lambda- ││││ control: OK │││└─── Idle │││ switch closed ││└──── A/C compr. ││ OFF │└───── Cat temp. │ reached └────── no DTC stored in │ memory 1: Conditions attained 0: conditions not attained |

| 002 | General |  | Systems with Mass Airflow Sensor |  |
| --- | --- | --- | --- | --- |
|  | RPM | Load | Mean injection time | Air mass |
|  | [1/min] | [%] | [ms] | [g/s] |
| 002 | General |  | Systems with intake manifold pressure sensor |  |
|  | RPM | Load | Mean injection time | Intake manifold pressure |
|  | [1/min] | [%] | [ms] | [mbar] |
| 003 | General |  | Systems with Mass Airflow Sensor |  |
|  | RPM | Air mass | Throttle valve angle (Potentiometer) | Ignition angle (actual) |
|  | [1/min] | [g/s] | [%] | [°KW] |
| 003 | General |  | Systems with intake manifold pressure sensor |  |
|  | RPM | Intake manifold pressure | Throttle valve angle (Potentiometer) | Ignition angle (actual) |
|  | [1/min] | [mbar] | [%] | [°KW] |
| 004 | General |  |  |  |
|  | RPM | Voltage | Cooling temperature | Intake air temperature |
|  | [1/min] | [V] | [°C] | [°C] |
| 005 | General |  |  |  |
|  | RPM | Load | Speed | Operating condition |
|  | [1/min] | [%] | [km/h] | Idle / partial load / full load / SA / BA |
| 006 | General |  |  |  |
|  | RPM | Load | Intake air temperature | Altitude correction |
|  | [1/min] | [%] | [°C] | [%] |
| 007 | General BDE |  |  |  |
|  | RPM | Load | Intake manifold pressure Mass Airflow Sensor | Pressure bake booster |
|  | [1/min] | [%] | [mbar] | [mbar] |
| 008 | General |  |  |  |
|  | Condition brake | Supply voltage | Condition vacuum pump for brake | Pressure bake booster |
|  | activated / not activated | [V] | Pump ON / Pump OFF | [mbar] |
| 009 |  |  |  |  |

- Groups 010 - 019:  Ignition & Misfire Detection

| 010 | Ignition |  |  |  |
| --- | --- | --- | --- | --- |
|  | RPM | Load | Throttle valve angle (Potentiometer) | Ignition angle (actual) |
|  | [1/min] | [%] | [%] | [°KW] |
| 011 | Ignition |  |  |  |
|  | RPM | Coolant temperature | Intake air temperature | Ignition angle (actual) |
|  | [1/min] | [°C] | [°C] | [°KW] |
| 012 | Distributor adjustment |  |  |  |
|  | RPM | Load | No. of crankshaft tooth at camshaft flank low->high | No. of crankshaft tooth at camshaft flank low->high |
|  | [1/min] | [%] | [xx] | [xx] |
| 013 |  |  |  |  |
| 014 | Misfire recognition |  |  |  |
|  | RPM | Last | Misfire counter | Misfire recognition |
|  | [1/min] | [%] | [n] | Text: activated / locked |
| 015 | Misfire recognition |  |  |  |
|  | Counter Cyl.. 1 | Counter Cyl. 2 | Counter Cyl. 3 | Misfire recognition |
|  | [n] | [n] | [n] | Text: activated / locked |
| 016 | Misfire recognition |  |  |  |
|  | Counter Cyl.. 4 | Counter Cyl. 5 | Counter Cyl. 6 | Misfire recognition |
|  | [n] | [n] | [n] | Text: activated / locked |
| 017 | Misfire recognition |  |  |  |
|  | Counter Cyl.. 7 | Counter Cyl. 8 | Counter Cyl. 9 | Misfire recognition |
|  | [n] | [n] | [n] | Text: activated / locked |
| 018 | Load / RPM window misfire recognition (if no misfire recognized: 0 at all positions) |  |  |  |
|  | Lower RPM limit | Upper RPM limit | Lower RPM limit | Upper RPM limit |
|  | [n] | [n] | [%] | [%] |
| 019 | Misfire recognition |  |  |  |
|  | Counter Cyl. 10 | Counter Cyl. 11 | Counter Cyl. 12 | Misfire recognition |
|  | [n] | [n] | [n] | Text: activated / locked |

- Groups 020 - 029: Knock Control

| 020 | Knock control |  |  |  |
| --- | --- | --- | --- | --- |
|  | Ignition angle retard Cyl. 1 | Ignition angle retard Cyl. 2 | Ignition angle retard Cyl. 3 | Ignition angle retard Cyl.4 |
|  | [°KW] | [°KW] | [°KW] | [°KW] |
| 021 | Knock control |  |  |  |
|  | Ignition angle retard Cyl. 5 | Ignition angle retard Cyl. 6 | Ignition angle retard Cyl. 7 | Ignition angle retard Cyl. 8 |
|  | [°KW] | [°KW] | [°KW] | [°KW] |
| 022 | Knock control |  |  |  |
|  | RPM | Load | Ignition angle retard Cyl. 1 | Ignition angle retard Cyl. 2 |
|  | [1/min] | % | [°KW] | [°KW] |
| 023 | Knock control |  |  |  |
|  | RPM | Last | Retard Cyl. 3 | Retard Cyl. 4 |
|  | [1/min] | [%] | [°KW] | [°KW] |
| 024 | Knock control |  |  |  |
|  | RPM | Last | Retard Cyl. 5 | Retard Cyl. 6 |
|  | [1/min] | [%] | [°KW] | [°KW] |
| 025 | Knock control |  |  |  |
|  | RPM | Last | Retard Cyl. 7 | Retard Cyl. 8 |
|  | [1/min] | [%] | [°KW] | [°KW] |
| 026 | Knock control, knock sensor voltage (amplifier factor included) |  |  |  |
|  | Cyl.. 1 | Cyl.. 2 | Cyl.. 3 | Cyl.. 4 |
|  | [V] | [V] | [V] | [V] |
| 027 | Knock control, knock sensor voltage (amplifier factor included) |  |  |  |
|  | Cyl.. 5 | Cyl.. 6 | Cyl.. 7 | Cyl.. 8 |
|  | [V] | [V] | [V] | [V] |
| 028 | Test knock sensor, short trip |  |  |  |
|  | RPM | Load | Coolant temperature | Result |
|  | [1/min] | [%] | [°C] | Text: Test ON / Test OFF / Sys. OK / Sys. not OK |
| 029 |  |  |  |  |

- Groups 030-049:  Mixture Control, O2S Heating, Catalytic Converter

| 030 | Oxygen sensor, status |  | 2-bank systems |  |
| --- | --- | --- | --- | --- |
|  | Bank 1, sensor 1 | Bank 1, sensor 2 | Bank 2, sensor 1 | Bank 2, sensor 2 |
|  | xxx ││└ Control active │└─ Sensor ready └── Sensor heat ON | xxx ││└ Control active │└─ Sensor ready └── Sensor heat ON | xxx ││└ Control active │└─ Sensor ready └── Sensor heat ON | xxx ││└ Control active │└─ Sensor ready └── Sensor heat ON |
| 030 | Oxygen sensor, status |  | 1- bank systems |  |
|  | Bank 1, sensor 1 | Bank 1, sensor 2 |  |  |
|  | xxx ││└ Control active │└─ Sensor ready └── Sensor heat ON | xxx ││└ Control active │└─ Sensor ready └── Sensor heat ON |  |  |
| 031 | Oxygen sensor voltage |  | 2- bank systems |  |
|  | Bank 1, sensor 1 | Bank 1, sensor 2 | Bank 2, sensor 1 | Bank 2, sensor 2 |
|  | [V] | [V] | [V] | [V] |
| 031 | Oxygen sensor voltage |  | 1- bank systems |  |
|  | Bank 1, sensor 1 | Bank 1, sensor 2 |  |  |
|  | [V] | [V] |  |  |
| 031 | Linear oxygen sensors |  | 2- bank systems |  |
|  | Lambda actual value Bank 1 | Lambda specified value Bank 1 | Lambda actual value Bank 2 | Lambda specified value Bank 2 |
|  | [] | [] | [] | [] |
| 031 | Linear oxygen sensors |  | 1- bank systems |  |
|  | Lambda actual value Bank 1 | Lambda specified value Bank 1 |  |  |
|  | [] | [] |  |  |
| 032 | Oxygen sensors learning values (maximum value) |  | 2- bank systems |  |
|  | Bank 1, sensor 1, idle | Bank 1, sensor 1, partial load | Bank 2, sensor 1, idle | Bank 2, sensor 1, partial load |
|  | [%] | [%] | [%] | [%] |
| 032 | Oxygen sensors learning values (maximum value) |  | 1- bank systems |  |
|  | Bank 1, sensor 1, idle | Bank 1, sensor 1, partial load |  |  |
|  | [%] | [%] |  |  |

| 033 | Lambda control value |  | 2- bank systems |  |
| --- | --- | --- | --- | --- |
|  | Bank 1, control value | Bank 1, Oxygen sensor voltage | Bank 2, control value | Bank 2, knock sensor voltage |
|  | [%] | [V] | [%] | [V] |
| 033 | Lambda control value |  | 1- bank systems |  |
|  | Bank 1, control value | Bank 1, Oxygen sensor voltage |  |  |
|  | [%] | [V] |  |  |
| 033 | Linear oxygen sensor control value |  | 2- bank systems |  |
|  | Bank 1, control value | Bank 1, sensor voltage before cat. converter of a broad band sensor | Bank 2, control value | Bank 2, sensor voltage before cat. converter of a broad band sensor |
|  | [%] | [V] | [%] | [V] |
| 034 | Oxygen sensor aging test Bank 1 or Bank 3 before catalytic converter, short trip |  |  |  |
|  | RPM | Exhaust gas / cat. converter temperature | Length of period | Result |
|  | [1/min] | [°C] | [s] | Text: Test ON / Test OFF / B1-S1 OK / B1-S1 not OK or: B3-S1 OK / B3-S1 not OK |
| 034 | Oxygen sensor aging test Bank 1 or Bank 3 before cat. converter for linear oxygen sensors. short trip |  |  |  |
|  | RPM | Exhaust gas / cat. converter temperature | Dynamic factor | Result |
|  | [1/min] | [°C] | [ ] | Text: Test ON / Test OFF / B1-S1 OK / B1-S1 not OK or B3-S1 OK / B3-S1 not OK |
| 035 | Oxygen sensor aging test Bank 2 or Bank 4 before cat. converter, short trip |  | 2-bank systems |  |
|  | RPM | Exhaust gas / cat. converter temperature | Length of period | Result |
|  | [1/min] | [°C] | [s] | Text: Test ON / Test OFF / B2-S1 OK / B2-S1 not OK or B4-S1 OK / B4-S1 not OK |
| 035 | Oxygen sensor aging test Bank 2 or Bank 4 before cat. converter for linear oxygen sensors, short trip |  | 2- bank systems |  |
|  | RPM | Exhaust gas / cat. converter temperature | Dynamic factor | Result |
|  | [1/min] | [°C] | [ ] | Text: Test ON / Test OFF / B2-S1 OK / B2-S1 not OK or: B4-S1 OK / B4-S1 not OK |
| 035 | Oxygen sensor aging test Bank 2 before cat. converter |  | 1- bank systems |  |

| 036 | Oxygen sensor readiness after cat. converter, short trip |  | 2- bank systems |  |
| --- | --- | --- | --- | --- |
|  | Bank 1, sensor 2 | Result | Bank 2, sensor 2 | Result |
|  | Sensor voltage | Text: Test ON / Test OFF / B1-S2 OK / B1-S2 not OK or: B3-S2 OK / B3-S2 not OK | Sensor voltage | Text: Test ON / Test OFF / B2-S2 OK / B2-S2 not OK or: B4-S2 OK / B4-S2 not OK |
| 036 | Oxygen sensor readiness after cat. converter, short trip |  | 1- bank systems |  |
|  | Bank 1, sensor 2 | Result |  |  |
|  | Sensor voltage | Text: Test ON / Test OFF / B1-S2 OK / B1-S2 not OK. |  |  |
| 037 | Oxygen sensors, short trip |  |  |  |
|  | Last | Bank 1: Oxygen sensor voltage after cat. converter | Bank 1: TV shift | Result |
|  | [%] | [V] | [ms] | Text: Test ON / Test OFF / Sys. OK / Sys. not OK |

| 037 | Linear oxygen sensors, short trip |  |  |  |
| --- | --- | --- | --- | --- |
|  | Last | Bank 1: Oxygen sensor voltage after cat. converter | Bank 1: D Lambda | Result |
|  | [%] | [V] | [ ] | Text: Test ON / Test OFF / Sys. OK / Sys. not OK |
| 038 | Oxygen sensors, short trip |  | 2- bank systems |  |
|  | Last | Bank 2: Oxygen sensor voltage after cat. converter | Bank 2: TV shift | Result |
|  | [%] | [V] | [ms] | Text: Test ON / Test OFF / Sys OK / Sys. not OK |
| 038 | Linear oxygen sensors, short trip |  | 2- bank systems |  |
|  | Last | Bank 2: Oxygen sensor voltage after cat. converter | Bank 2: D Lambda | Result |
|  | [%] | [V] | [ ] | Text: Test ON / Test OFF / Sys OK / Sys. not OK |
| 038 | Oxygen sensors |  | 1- bank systems |  |
|  | Not used |  |  |  |
| 039 | Sensor exchange after cat. converter short trip |  |  |  |
|  | Air mass | Bank 1: sensor voltage | Bank 2 sensor voltage | Result |
|  | [g/s] | [V] | [V] | Text: Test ON / Test OFF / Sys OK / Sys. not OK |

| 040 | Oxygen sensor heaters resistor Combined wires and heaters |  | 2- bank systems |  |
| --- | --- | --- | --- | --- |
|  | Heater resistor | Condition | Heater resistors | Condition |
|  | Banks 1+2, sensor 1 [Ω] | Text: Heater before cat. ON/ Heater before cat. OFF | Banks 1+2, sensor 2 [Ω] | Text: Heater before cat. ON/ Heater before cat. OFF |
| 040 |  |  | 1- bank systems |  |
|  | Not used |  |  |  |
| 041 | Oxygen sensor heater |  | 1- bank systems |  |
|  | Resistor | Condition or duty cycle | Resistors | Condition |
|  | Bank 1, sensor 1 [Ω] | Text: Heater before cat. ON/ Heater before cat. OFF or: sensor 1 [%] | Bank 1, Sensor 2 [Ω] | Text: Heater after cat. ON / Heater after cat. OFF |
| 042 | Oxygen sensor heaters, separate wires of heaters |  |  | 2- bank systems |
|  | Resistor | Condition or duty cycle | Resistors | Condition |
|  | Bank 2, Sensor 1 [Ω] | Text: Heater before cat. ON/ Heater before cat. OFF or: Sensor 1 [%] | Bank 2, Sensor 2 [Ω] | Text: Heater after cat. ON/ Heater after cat. OFF |
| 043 | Oxygen sensor ageing after cat. converter, linear oxygen sensors, short trip |  |  | Bank 1 |
|  | RPM | Exhaust gas / cat. converter temperature | Oxygen sensor voltage | Result |
|  | [U/min] | [°C] | [V] | Text: Test ON / Test OFF / B1-S2 OK / B1-S2 not OK Or: B3-S2 OK / B3-S2 not OK |
| 044 | Oxygen sensor ageing after cat. converter, short trip |  |  | Bank 2 |
|  | RPM | Exhaust gas / cat. converter temperature | Oxygen sensor voltage | Result |
|  | [U/min] | [°C] | [V] | Text : Test ON / Test OFF / B2-S2 OK / B2-S2 not OK Or: B4-S2 OK / B4-S2 not OK |
| 044 |  |  |  | 1-bank systems |
| 045 | NOx memory cat. converter Bank 1, short trip |  | NOx memory cat. converter Bank 2 |  |
|  | Factor memory entry NOx | Result | Factor memory entry NOx | Result |
|  |  | Text: Test ON / Test OFF / Sys. OK / Sys. not OK |  | Text: Test ON / Test OFF / Sys. OK / Sys. not OK |

| 046 | Cat. converter conversion test Bank 1 or Bank 3, short trip |  |  |  |
| --- | --- | --- | --- | --- |
|  | RPM | Cat. converter temperature | Measuring value cat. Converter conversion | Result |
|  | [1/min] | [°C] |  | Text: Test ON / Test OFF / Cat B1 OK / Cat B1 not OK or: Cat B3 OK / Cat B3 not OK |
| 047 | Cat. converter conversion test Bank 2 or Bank 4, short trip |  | 2-Bank-Systems |  |
|  | RPM | Cat. converter temperature | Measuring value cat. Converter conversion | Result |
|  | [1/min] | [°C] |  | Text: Test ON / Test OFF / Cat B2 OK / Cat B2 not OK or: Cat B4 OK / Cat B4 not OK |
| 047 | Cat. converter conversion test Bank 2 |  | 1-Bank-Systems |  |
|  | Not used |  |  |  |
| 048 | Thermal cat. converter diagnosis Bank 1, short trip |  |  |  |
|  | Operating system BDE | Number of test steps | Exothermal temperature increase | Result |
|  | [-] | [-] | [K] | Text: Test ON / Test OFF / Sys. OK / Sys. not OK |
| 049 | Thermal cat. converter diagnosis Bank 2, short trip |  |  |  |
|  | Operating system BDE | Number of test steps | Exothermal temperature increase | Result |
|  | [-] | [-] | [K] | Text: Test ON / Test OFF / Sys. OK / Sys. not OK |

- Groups 050 - 059:  Engine Speed Control

| 050 | RPM increase |  | Systems without separation for rear window defroster /A/C readiness |  |
| --- | --- | --- | --- | --- |
|  | RPM actual value | RPM specified value | Rear window defroster/ Request for A/C | A/C compressor |
|  | [1/min] | [1/min] | Text ON / OFF | Text ON / OFF / decrease |
| 050 | RPM increase |  | Systems with separation for rear window defroster /A/C readiness |  |
|  | RPM actual value | RPM specified value | Request for A/C | A/C compressor |
|  | [1/min] | [1/min] | Text ON / OFF | Text ON / OFF / decrease |
| 051 | RPM, shift initiations |  |  |  |
|  | RPM actual value | RPM specified value | Driving range (only for automatic transmission) | Supply voltage |
|  | [1/min] | [1/min] | 0-6 0 - Idle 1-6 - driving ranges | [V] |
| 052 | RPM increase |  | Systems without separation for rear window defroster /A/C readiness |  |
|  | Not used |  |  |  |
| 052 | RPM increase |  | Systems with separation for rear window defroster /A/C readiness |  |
|  | RPM (actual value) | RPM (specified value) | A/C readiness | Rear window defroster |
|  | [1/min] | [1/min] | Text ON / OFF | Text ON / OFF |
| 053 | RPM increase based on generator load |  |  |  |
|  | RPM (actual value) | RPM (specified value) | Voltage | Generator load |
|  | [1/min] | [1/min] | [V] | [%] |
| 054 | Closed throttle position switch |  |  |  |
|  | RPM | Operating conditions | Throttle valve angle (Potentiometer) | Throttle valve angle (Potentiometer) |
|  | [1/min] | (LL, TL, VL, SA, BA) | [%] | [%] |
| 054 | Closed throttle position switch for E-Gas-systems |  |  |  |
|  | RPM | Operating conditions | Sensor 1 for accelerator pedal position | Throttle valve angle (Potentiometer) |
|  | [1/min] | (LL, TL, VL, SA, BA) | [%] | [%] |

| 055 | Idle air control |  | Systems without separation for rear window defroster /A/C readiness |  |
| --- | --- | --- | --- | --- |
|  | RPM | Idle air control | Current learning value for idle air control | Operating conditions |
|  | [1/min][1] |  |  | x0xxx ││││└─A/C ││││ compressor ON │││└──Driving │││ range selected ││└───A/C readiness ││ ON │└────always 0 └─────Steering to stop 1: Conditions attained 0: conditions not attained |
| 055 | Idle air control |  | Systems with separation for rear window defroster /A/C readiness |  |
|  | RPM | Idle air control | Learning value for idle air control | Operating conditions |
|  | [1/min] |  |  | xxxxx ││││└─A/C ││││ compressor ON │││└──Driving │││ range selected ││└───A/C readiness ││ ON │└────rear window │ defroster ON └─────Steering to stop 1: Conditions attained 0: conditions not attained |

| 056 | Idle air control |  | Systems with separation for rear window defroster /A/C readiness |  |
| --- | --- | --- | --- | --- |
|  | RPM (actual value) | RPM (specified value) | Idle air control valve | Operating conditions |
|  | [1/min] | [1/min] |  | x0xxx ││││└─A/C ││││ compressor ON │││└──Driving │││ range selected ││└───A/C readiness ││ ON │└────always 0 └─────Steering to stop 1: Conditions attained 0: conditions not attained |

| 056 | Idle air control |  | Systems without separation for rear window defroster /A/C readiness |  |
| --- | --- | --- | --- | --- |
|  | RPM (actual value) | RPM (specified value) | Idle air control valve | Operating conditions |
|  | [1/min] | [1/min] | [[8]] | xxxxx ││││└─A/C ││││ compressor ON │││└──Driving │││ range selected ││└───A/C readiness ││ ON │└────rear window │ defroster ON └─────Steering to stop 1: Conditions attained 0: conditions not attained |
| 057 | Idle air control, pressure signal A/C compressor |  |  |  |
|  | RPM (actual value) | RPM (specified value) | A/C compressor | Duty cycle pressure receiver |
|  | [1/min] | [1/min] | Text: ON / OFF /decreasing | [%] |
| 058 |  |  |  |  |
|  | RPM | Load | Engine bearing 1 right | Engine bearing 2 left |
|  | [1/min] | [%] | ON / OFF | ON / OFF |
| 059 |  |  |  |  |

- Groups 060 - 069:  Throttle Valve Control

| 060 | ESB adaption |  | Vehicles with ESB |  |
| --- | --- | --- | --- | --- |
|  | Throttle valve angle (potentiometer) | Throttle valve angle (potentiometer) | Operating condition | Adaption condition |
|  | [%] | [%] | Text: Idle / partial load / full load / deceleration enrichment / acceleration enrichment | Text: ADP runs / ADP OK / ERROR |
| 060 | E-Gas adaption |  | Vehicles with E-Gas |  |
|  | Throttle valve angle (potentiometer 1) | Throttle valve angle (potentiometer 2) | Electrically controlled throttle valve adaption status | Operating condition |
|  | [%] | [%] | [n] | Text: ADP runs / ADP OK / ERROR |
| 061 | ESB/E-Gas |  | Systems without separation for rear window defroster /A/C readiness |  |
|  | RPM | Supply voltage Ubat | Triggering throttle valve control | Operating condition |
|  | [1/min] | [V] | [%] if possible | 0xxx │││└─ A/C │││ compressor ON ││└── driving ││ range selected │└─── rear window │ defroster ON └──── always 0 |
| 061 | ESB/E-Gas |  | Systems with separation for rear window defroster /A/C readiness |  |
|  | RPM | Supply voltage Ubat | Actuating throttle valve control | Operating condition |
|  | [1/min] | [V] | [%] | xxxx │││└─A/C │││ compressor ON ││└──driving ││ range selected │└───A/C readiness │ ON └────Rear window │ defroster ON |
| 062 | E-Gas, potentiometer voltage relationship U/Uref |  |  |  |
|  | Angle sensor 1 for throttle valve drive (0->100%) | Angle sensor 2 for throttle valve drive (0->100%) | Sensor 1 accelerator pedal position (0->100%) | Sensor 2 accelerator pedal position (0->100%) |
|  | [%] | [%] | [%] | [%] |

| 063 | Kick-down adaption |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sensor 1 accelerator pedal position (0->100%) | Sensor 1 learned Kick-down point (0->100%) | Switch |  |  |  | Result |  |
|  | [%] | [%] | Kick-down |  |  |  | Actuated / ADP runs / ADP OK / ERROR |  |
| 064 | Throttle valve potentiometer adaption values |  |  |  |  |  |  |  |
|  | Potentiometer 1 lower adaption | Potentiometer 2 1 lower adaption | Emergency air gap potentiometer 1 |  |  |  | Emergency air gap potentiometer 2 |  |
|  | [V] | [V] | [V] |  |  |  | [V] |  |
| 065 |  |  |  |  |  |  |  |  |
| 066 | Cruise control switched free for 4 position operating lever |  |  |  |  |  |  |  |
|  | Vehicle speed actual | Switch positions |  | Vehicle speed specified |  | Switch positions For 4 position operating lever |  |  |
|  | [km/h] | xxxxxxxx │││││││└ brake light switch │││││││ Pedal pressed ││││││└ brake pedal switch ││││││ Pedal pressed │││││└─ clutch switch │││││ Pedal pressed ││││└── CC function ││││ switched free │││└─── ADR function │││ switched free ││└──── not used ││ 00 ADR/ CC not active │ │ (also used for not installed) 01 ADR/ CC in control mode 10 Condition „over-controlled“, │ driver accelerates more │ │ than cruise control regulator 11 ADR not released 1: Conditions attained 0: Conditions not attained |  | [km/h] in 1 km/h-steps (only displayed when cruise control function is freely switched) |  | xxxxxxxx │││││││└ CC ON / OFF │││││││ ││││││└ memory set ││││││ │││││└─ Set / tip down / │││││ decelerate ││││└── Resume ││││ Tip up / acceleration ----0000 CC engaged OFF ----0001 CC tipped OFF ----0011 CC engaged ON │ (stand by) ----01x1 Set/Tip down/ decelerate ----1011 Resume/Tip up/ │ │ accelerate |  |  |

| 066 | Cruise control switched free for 6 position operating lever |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Vehicle speed actual | Switch positions | Vehicle speed specified | Switch positions For 6 position operating lever |  |  |
|  | [km/h] | xxxxxxxx │││││││└ brake light switch │││││││ Pedal pressed ││││││└ brake pedal switch ││││││ Pedal pressed │││││└─ clutch switch │││││ Pedal pressed ││││└── CC function ││││ switched free │││└─── ADR function │││ switched free ││└──── not used ││ 00 ADR/ CC not active │ │ (also used for not installed) 01 ADR/ CC in control mode 10 Condition „over-controlled“, │ driver accelerates more │ │ than cruise control regulator 11 ADR not released 1: Conditions attained 0: Conditions not attained | [km/h] in 1 km/h-steps (is only issued with freely switched cruise control function) | xxxxxxxx │││││││└ CC ON/OFF engaged │││││││ ││││││└ (CAN) ││││││ CC tipping yes/no │││││└─ memory set │││││ ││││└── Tip down / decelerate ││││ │││└─── Tip up / accelerate │││ ││└──── Set ││ │└───── Resume │ └────── CC ON/OFF engaged (Hardware PIN) 0-000000 CC engaged OFF 1-000001 CC engaged ON │ (stand-by) 1-000011 CC tipped OFF 1-000101 Tip down/decelerate 1-001001 Tip up/accelerate 0-010000 Set │ │ (with CC engaged OFF) 1-010001 Set │ │ (with CC engaged ON) 1-100001 Resume |  |  |
| 067 |  |  |  |  |  |  |
| 068 |  |  |  |  |  |  |
| 069 |  |  |  |  |  |  |

- Groups 070 - 079: Emission Control Systems (EVAP, Sec. AIR, EGR)

| 070 | Fuel tank venting and valve testing, short trip |  |  |  |
| --- | --- | --- | --- | --- |
|  | Opening degree fuel tank ventilation | Lambda regular/ diagnostic value with active diagnose | Idle air control valve / diagnostic value with active diagnose | Result |
|  | [%] | [%] | [%] / [g/sec] | Test ON / Test OFF / Fuel tank ventilation OK / Fuel tank ventilation not OK. |
| 071 | Fuel tank leak test, short trip |  |  |  |
|  | Condition reed contact | DTC | Test status | Result |
|  | Text Reed Open / Reed Closed | Text Small leak/large leak | System test/measurement/ measurement end | Test ON / Test OFF / Abort Syst. OK / Syst. Not OK |
| 072 | Tank leak test |  |  |  |
| 073 |  |  |  |  |
| 074 | EGR regulator solenoid adaption |  |  |  |
|  | Null-Position | Max. stop | Current potentiometer value | Text |
|  | [V] | [V] | [V] | Leak detection pump runs / LDP OK / ERROR |
| 075 | EGR, short trip |  | Systems with Temp.-sensor |  |
|  | Engine RPM | EGR-Temp. Sensor | EGR temperature difference | Result |
|  | [1/min] | [°C] | [°C] | Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 075 | EGR, short trip |  | Systems with intake manifold pressure sensor |  |
|  | Engine RPM | Intake manifold pressure | Intake manifold pressure difference | Result |
|  | [1/min] | [mbar] | [mbar] | Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 075 | EGR, for map adaption short trip |  | Systems with intake manifold pressure sensor |  |
|  | Pressure difference diagnostics EGR phase 1 and 2 | Pressure difference diagnostics EGR phase 2 and 3 | Pressure difference diagnostics EGR phase 1 and 3 | Result |
|  | [hPa] | [hPa] | [hPa] | Test ON / Test OFF / Syst. OK / Syst. Not OK |

| 076 | EGR for pressure systems |  |  |  |
| --- | --- | --- | --- | --- |
|  | Engine RPM | Intake manifold pressure | Opening degree (U/Uref) EGR potentiometer | Duty cycle EGR valve |
|  | [1/min] | [mbar] | [%] | [%] |
| 076 | EGR for air mass system |  |  |  |
|  | Engine RPM | Last | Opening degree (U/Uref) EGR potentiometer | Duty cycle EGR valve |
|  | [1/min] | [%] | [%] | [%] |
| 076 | EGR map adaption |  |  |  |
|  | Actual EGR value – potentiometer without offset | Correction factor in the upper opening range | Correction factor in the lower opening range | Status |
|  | [V] | [%] | [%] | Test OFF / Leak detection pump runs / LDP OK / ERROR |
| 077 | Test secondary air injection system, short trip, Bank 1 |  | Systems with conventional oxygen sensor |  |
|  | RPM | Engine air mass | Air mass secondary air injection system | Result |
|  | [1/min] | [g/sec] | [g/sec] | Test ON / Test OFF / Abort Syst. OK / Syst. Not OK |
| 077 | Test secondary air injection system, short trip, Bank 1 |  | Systems with linear oxygen sensor |  |
|  | RPM | Engine air mass | relative Air mass | Result |
|  | [1/min] | [g/sec] | [ ] | Test ON / Test OFF / Abort Syst. OK / Syst. Not OK |
| 078 | Test secondary air injection system, short trip Bank 2 |  | Systems with conventional oxygen sensor |  |
|  | RPM | Engine air mass | Air mass secondary air injection system | Result |
|  | [1/min] | [g/sec] | [g/sec] | Test ON / Test OFF / Abort Syst. OK / Syst. Not OK |
| 078 | Test secondary air injection system, short trip, Bank 2 |  | Systems with linear oxygen sensor |  |
|  | RPM | Engine air mass | Relative air mass | Result |
|  | [1/min] | [g/sec] | [ ] | Test ON / Test OFF / Abort Syst. OK / Syst. Not OK |
| 079 |  |  |  |  |

- Groups 080-085: Control module identification

| 080 | Control module identification (display shown at NA-No. 63) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Manufacturer’s code and marking | Manufacturing date |  | Manufacture’s change status |  | Manufacturer’s test stand number. | Manufacturer’s running number. |
|  | hhh-kkk | dd.mm.yy |  | 12345678 |  | pppp | nnnn |
| 081 | Control module identification (display shown at NA-No. 63) |  |  |  |  |  |  |
|  | Vehicle identification number |  | Limit No or serial number |  | Type test number |  |  |
|  | WVWZZZ1JZWW12345 |  | 12345678901234 |  | 1234567 |  |  |
| 082 | Control module identification (display shown at NA-No. 63) |  |  |  |  |  |  |
|  | Flash tool code (FTC) | Flash date |  | Hardware Comp. group type |  | Software Comp. group type |  |
|  | 1234567890123 | dd.mm.yy |  | 123 | 12 | 123 | 12 |
| 083 | Control module identification |  |  |  |  |  |  |
| 084 | Control module identification |  |  |  |  |  |  |
| 085 | Control module identification |  |  |  |  |  |  |

- Groups 090 - 098 Camshaft & Intake Manifold Control

| 090 | Camshaft adjustment Intake |  |  |  |
| --- | --- | --- | --- | --- |
|  | RPM | Adjustment | Adjustment Bank 1 | Adjustment Bank 2 |
|  | [1/min] | Text ON / OFF | [°KW] | [°KW] |
| 090 | Camshaft adjustment Exhaust |  |  |  |
|  | RPM | Adjustment | Adjustment Bank 1 | Adjustment Bank 2 |
|  | [1/min] | Text ON/OFF | [°KW] | [°KW] |
| 090 | Continuous camshaft adjustment Exhaust Bank 1 |  |  |  |
|  | RPM | Duty cycle | Adjustment specified | Adjustment actual |
|  | [1/min] | [%] | [°KW] | [°KW] |
| 091 | Camshaft adjustment Intake Bank 1 |  |  |  |
|  | RPM | Last | Adjustment | Adjustment |
|  | [1/min] | [%] | Text ON / OFF | [°KW] |
| 091 | Continuous camshaft adjustment, Intake Bank 1 |  |  |  |
|  | RPM | Duty cycle | Adjustment specified | Adjustment actual |
|  | [1/min] | [%] | [°KW] | [°KW] |
| 092 | Camshaft adjustment Intake Bank 2 |  |  |  |
|  | RPM | Last | Adjustment | Adjustment |
|  | [1/min] | [%] | Text ON / OFF | [°KW] |
| 092 | Continuous camshaft adjustment, Intake Bank 2 |  |  |  |
|  | RPM | Duty cycle | Adjustment specified | Adjustment actual |
|  | [1/min] | [%] | [°KW] | [°KW] |
| 093 | Camshaft adaption values Intake |  | 2-Bank Systems |  |
|  | RPM | Last | Phase position [1] Bank 1[2] | Phase position Bank 2 |
|  | [1/min] | [%] | [°KW] | [°KW] |
| 093 | Continuous camshaft adaption values |  |  |  |
|  | Phase position intake Bank 1 | Phase position intake Bank 2 | Phase position exhaust Bank 1 | Phase position exhaust Bank 2 |
|  | [°KW] [3] | [°KW] | [°KW] | [°KW] |
| 094 | Camshaft adjustment Banks 1 and 2 Intake, short trip |  |  |  |
|  | RPM | Camshaft adjustment | Test result Bank 1 | Test result Bank 2 |
|  | [1/min] | Text: Camshaft position ON / Camshaft position OFF | Test ON / Test OFF / Syst. OK / Syst. Not OK | Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 094 | Continuous camshaft adjustment Banks 1 and 2 intake, short trip |  |  |  |
|  | RPM | Phase position intake | Test result Bank 1 | Test result Bank 2 |
|  | [1/min] | [°KW] | Test ON / Test OFF / Syst. OK / Syst. Not OK | Test ON / Test OFF / Syst. OK / Syst. Not OK |

| 095 | Intake manifold change-over |  | Single step change-over |  |
| --- | --- | --- | --- | --- |
|  | Engine RPM | Last | Coolant temperature | Status |
|  | [1/min] | [%] | [°C] | Text: ein / aus |
| 095 | Intake manifold change-over |  | Multi-step change-over |  |
|  | Engine RPM | Last | Coolant temperature | Status |
|  | [1/min] | [%] | [°C] | Text: Off / step 1 / step 2 |
| 096 | Camshaft adjustment Banks 1 and 2 Exhaust, short trip |  |  |  |
|  | RPM | Camshaft adjustment | Test result Bank 1 | Test result Bank 2 |
|  | [1/min] | Text: Camshaft position ON / Camshaft position OFF | Test ON / Test OFF / Syst. OK / Syst. Not OK | Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 096 | Continuous camshaft adjustment Banks 1 and 2 Exhaust, short trip |  |  |  |
|  | RPM | Phase position exhaust | Test result Bank 1 | Test result Bank 2 |
|  | [1/min] | [°KW] | Test ON / Test OFF / Syst. OK / Syst. Not OK | Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 097 | Intake air change-over / Snow flap |  |  |  |
|  | RPM | Last | Temperature / Pressure | Intake air change-over |
|  | [1/min] | [%] | [°C]/[mbar] | ON/ OFF |
| 098 | Continuous camshaft adjustment Exhaust Bank 2 |  |  |  |
|  | RPM | Duty cycle | Adjustment specified | Adjustment actual |
|  | [1/min] | [%] | [°KW] | [°KW] |

- Groups 099 & 100: Compatibility Groups

| 099 | Shut-off l-Regulation (via basic setting) (for compatibility reasons with older systems) |  |  |  |
| --- | --- | --- | --- | --- |
|  | RPM | Coolant temperature | Lambda regulation | Lambda regulation |
|  | [1/min] | [°C] | [%] | ON/OFF |
| 100 | Readiness code (for compatibility reasons with older systems) |  |  |  |
|  | Ready bits (concluded tests) 1= not concluded 0= concluded | Coolant temperature | Time since engine start | OBD-Status |
|  | xxxxxxx │││││││└ Cat. │││││││ converters ││││││└ Heat cat │││││└ AKF-System ││││└ SL-System │││└ A/C ││└ Oxygen sensors │└ Oxygen sensor heating └ EGR | [°C] |  | xxxxxxxx │││││││└ no warm- │││││││ up cycle │││││││ possible ││││││└ warm-up ││││││ cycle finished │││││└ not used ││││└ not used │││└ at least one DTC │││ recognized ││└ Trip complete │└ Driving cycle fulfilled └ MIL ON 1: Description fulfilled 0: Description not fulfilled |

- Groups 101 - 109: Fuel Injection

| 101 | Fuel injection |  | Systems with Air mass measurement |  |
| --- | --- | --- | --- | --- |
|  | RPM | Last | Mean injection time (at deceleration = 0) | Air mass |
|  | [1/min] | [%] | [ms] | [g/s] |
| 101 | Fuel injection |  | Systems with intake manifold pressure measurement |  |
|  | RPM | Last | Mean injection time (at deceleration = 0) | Intake manifold pressure |
|  | [1/min] | [%] | [ms] | [mbar] |
| 102 | Fuel injection |  |  |  |
|  | RPM | Coolant temperature | Intake air temperature | Mean injection time |
|  | [1/min] | [°C] | [°C] | [ms] |
| 103 | Air-shrouded fuel injectors, short trip |  |  |  |
|  | RPM | Air mass idle air control | LFR-Offset adaptation | Results |
|  | [1/min] | [g/s] | [n] | Test ON / Test OFF / l injector OK / l injector not OK |
| 104 | Start adaptation values |  |  |  |
|  | Start engine temperature | Temperature adaptation factor 1 | Temperature adaptation factor 2 | Temperature adaptation factor 3 |
|  | [°C] | [%] | [%] | [%] |
| 105 | Cylinder shut off |  |  |  |
|  | RPM (actual) | Last | Coolant temperature | Shut-off |
|  | [U/min] | [%] | [°C] | Text: ON / OFF |
| 106 |  |  |  |  |
| 107 | Lambda regulation, short trip |  |  |  |
|  | RPM | Lambda regulation Bank 1 (average value) | Lambda regulation Bank 2 (average value) | Result |
|  | [1/min] | [%] | [%] | Text Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 108 |  |  |  |  |
| 109 |  |  |  |  |

- Groups 110 - 119:  Load Registration & Boost Pressure Control

| 110 | Load, full load enrichment |  |  |  |
| --- | --- | --- | --- | --- |
|  | RPM | Coolant temperature | Average injection time | Throttle valve angle (potentiometer) |
|  | [1/min] | [°C] | [ms] | [%] |
| 111 | Boost pressure control |  |  |  |
|  | Adaption value boost pressure control RPM range 1 | Adaption value boost pressure control RPM range 2 | Adaption value boost pressure control RPM range 3 | Adaption value boost pressure control RPM range 4 |
|  | [%] | [%] | [%] | [%] |
| 112 | Exhaust gas temperature |  |  |  |
|  | Exhaust gas temperature Bank 1 | Enrichment factor sensor Bank 1 | Exhaust gas temperature Bank 2 | Enrichment factor sensor Bank 2 |
|  | [°C] | [%] | [°C] | [%] |
| 113 | Last |  |  |  |
|  | RPM | Last | Throttle valve angle (potentiometer) | Air pressure |
|  | [1/min] | [%] | [%] | [mbar] |
| 114 | Boost pressure control |  |  |  |
|  | Specified load without correction | Specified load after correction | Actual load | Duty cycle Boost pressure valve |
|  | [%] | [%] | [%] | [%] |
| 115 | Boost pressure control |  |  |  |
|  | RPM | Last | Boost pressure specified value | Boost pressure actual value |
|  | [1/min] | [%] | [mbar] | [mbar] |
| 116 | Boost pressure control |  |  |  |
|  | RPM | Correction factor fuel | Correction factor coolant temperature | Correction factor intake air temperature |
|  | [1/min] | [%] | [%] | [%] |
| 117 | Boost pressure control |  |  |  |
|  | RPM | Gas pedal position | Throttle valve angle | Boost pressure specified |
|  | [1/min] | [%] | [%] | [mbar] |
| 118 | Boost pressure control |  |  |  |
|  | RPM | Intake air temperature | Duty cycle boost pressure control valve | Boost pressure before throttle valve |
|  | [1/min] | [°C] | [%] | [mbar] |
| 119 | Boost pressure control |  |  |  |
|  | RPM | Actual adaption value boost pressure control valve | Duty cycle boost pressure control valve | Boost pressure before throttle valve |
|  | [1/min] | [%] | [%] | [mbar] |

Groups 120 - 129: Communcations between Control Modules

| 120 | ASR/FDR |  |  |  |
| --- | --- | --- | --- | --- |
|  | Engine RPM | Specified moment ASR/FDR | Engine moment | Status |
|  | [1/min] | [Nm] | [Nm] | Text ASR active / ASR not active |
| 121 |  |  |  |  |
| 122 | Transmission |  |  |  |
|  | Engine RPM | Specified moment Transmission | Engine moment | Status |
|  | [1/min] | [Nm] | [Nm] | Text Engine intervention / no intervention |
| 123 | Reserved for CAN Bus signals |  |  |  |
| 124 | Reserved for CAN Bus signals |  |  |  |
| 125 | CAN Bus signals |  |  |  |
|  | see CAN-Product Description Manual |  |  |  |
|  | Transmission | ABS | Instrument cluster | A/C |
| 126 | CAN Bus signals |  |  |  |
|  | ADR 32 | LWS | Airbag | Electrical wiring |
| 127 | CAN Bus signals |  |  |  |
|  | All wheel | Level | Steering wheel |  |
| 128 | CAN Bus signals |  |  |  |
| 129 | CAN Bus signals |  |  |  |

- Groups 130-137: 	Engine cooling

| 130 | Map cooling (coolant filling), Short trip |  |  |  |
| --- | --- | --- | --- | --- |
|  | Temperature Engine outlet | Temperature Radiator outlet | Duty cycle Thermostat | Result |
|  | [°C] | [°C] | [%] | Text: Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 131 | Map cooling |  |  |  |
|  | Temperature Engine outlet | Temperature Engine outlet (specified) | Temperature Radiator outlet | Duty cycle Thermostat |
|  | [°C] | [°C] | [°C] | [%] |
| 132 | Map cooling |  |  |  |
|  | Temperature Radiator outlet (specified) | Temperature difference Engine and radiator outlet | Heater pre-run potentiometer | Status cooling |
|  | [°C] | [°C] | [%] | xxxxxxxx │││││││└ Error │││││││ in system ││││││└ Thermostat-││││││ actuation ││││││ active │││││└ Cooling fan │││││ triggering active ││││└ Control deviation ││││ 0 > specified temp. ││││ 1 < specified temp. │││└ Fan stage 2 active ││└ Fan stage 1 active │└ not used └ not used 1: Description fulfilled 0: Description not fulfilled |
| 133 |  |  |  |  |
| 134 | Temperature |  |  |  |
|  | Oil temperature | Ambient temperature | Intake air temperature | Engine outlet temperature |
|  | [°C] | [°C] | [°C] | [°C] |

| 135 | Coolant fan control, short trip |  |  |  |
| --- | --- | --- | --- | --- |
|  | Temperature Radiator outlet (specified) | Duty cycle Coolant fan actuation 1 | Duty cycle Coolant fan actuation 2 | Result |
|  | [°C] | [%] | [%] | Text: Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 136 | Relay for coolant fan actuation |  |  |  |
|  | Relay 1 | Relay 2 | Relay 3 | Relay 4 |
|  | ON / OFF | ON / OFF | ON / OFF | ON / OFF |
| 137 | A/C requirements |  |  |  |
|  | AC-inlet | Compressor | High pressure switch or Pressure of A/C system | Fan request from A/C system |
|  | ON / OFF | ON / OFF | ON / OFF or [bar] | [%] |

- Groups 140-147: BDE

| 140 | Pressure control valve, short trip |  |  |  |
| --- | --- | --- | --- | --- |
|  | Duty cycle DSV | Rail pressure specified | Rail pressure actual | Result |
|  | [%] | [bar] | [bar] | Text: Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 141 | Fuel supply system |  |  |  |
|  | Rail pressure regulator | Regulator rail pressure system | Regulator rail pressure system residual portion | Status rail pressure system |
|  | [bar] | [-] | [-] | [-] |
| 142 |  |  |  |  |
| 143 |  |  |  |  |
| 144 |  |  |  |  |
| 145 | Exhaust gas temperature sensor, short trip |  |  |  |
|  | Exhaust gas temperature from model Bank 1 | Exhaust gas temperature sensor measured Bank 1 | Exhaust gas temperature sensor measured Bank 2 | Result |
|  | [°C] | [°C] | [°C] | Text: Test ON / Test OFF / Syst. OK / Syst. Not OK |
| 146 |  |  |  |  |
| 147 | Charge movement flap for BDE |  |  |  |
|  | Charge movement flap Actual position | Charge movement flap Specified position | Offset value of potentiometer voltage LBK | Adaptation condition |
|  | [%] | [%] | [V] | Text ADP runs / ADP OK / Error (in emergency running mode) |

- Groups 160-169: Lambda control / SULEV-ULEV

| 160 | Individual cylinder recognition/ Individual cylinder control |  |  |  |
| --- | --- | --- | --- | --- |
|  | Normal Lambda control starting point cylinder 1 | Normal Lambda control starting point cylinder 2 | Normal Lambda control starting point cylinder 3 | Normal Lambda control starting point cylinder 4 |
|  | [-] | [-] | [-] | [-] |
| 161 | Individual cylinder recognition/ Individual cylinder control |  |  |  |
|  | Normal Lambda control starting point cylinder 5 | Normal Lambda control starting point cylinder 6 | Normal Lambda control starting point cylinder 7 | Normal Lambda control starting point cylinder 8 |
|  | [-] | [-] | [-] | [-] |
| 162 |  |  |  |  |
| 163 |  |  |  |  |
| 164 |  |  |  |  |
| 165 |  |  |  |  |
| 166 |  |  |  |  |
| 167 |  |  |  |  |
| 168 |  |  |  |  |
| 169 |  |  |  |  |